4-bit Parallel Binary Subtractor in Verilog

Overview

This project provides a Verilog implementation of a 4-bit parallel binary subtractor. It performs binary subtraction using two’s complement arithmetic by inverting the subtrahend, adding an initial borrow-in, and chaining four full adder modules. The design outputs both the 4-bit difference and a borrow-out flag indicating if the result is negative.

Features

- Subtracts two 4-bit binary numbers (A and B)

- Produces a 4-bit binary difference (Diff)

- Outputs a borrow-out (B_out) to indicate negative result (A < B)

- Modular code structure reusing a single-bit Full Adder module

- Handles positive, zero, and negative results (using two’s complement)

File List

- FullAdder1.v — 1-bit full adder module

- ParallelSub_4Bit.v — 4-bit parallel subtractor module (instantiates four FullAdder1s)

- PStb.v — Testbench with multiple test cases to verify the subtractor

How It Works

- The subtractor is built by connecting four 1-bit full adders in series.

- Each full adder operates on corresponding bits: the subtrahend bits are inverted, and an initial borrow-in (B_in) is provided for two’s complement logic.

- The final output includes the difference (Diff) and a borrow-out (B_out). B_out = 1 means the result is negative.

Example Output

Sample simulation output format (A, B, B_in | Difference, Borrow):

text

A = 0110 B = 0011 B_in = 1 | Difference = 0011 Borrow = 0  // 6 - 3 = 3

A = 0101 B = 1000 B_in = 1 | Difference = 1101 Borrow = 1  // 5 - 8 = -3 (two’s complement)

A = 0000 B = 0001 B_in = 1 | Difference = 1111 Borrow = 1  // 0 - 1 = -1 (two’s complement)

How to Run

- Copy all Verilog (.v) files to your project folder.

- Open your preferred Verilog simulator (ModelSim, Vivado, etc.).

- Compile all modules.

- Run the testbench (PStb.v) to see subtraction results for various inputs.

Customization

- To extend to more than 4 bits, instantiate additional full adder units in ParallelSub_4Bit.v.

- You can change or add more test cases in PStb.v as needed.

License

This project is open-source and free to use for educational purposes.
Fork, modify, and share as you wish!